# PetWatch SmartBowl S3

## End-to-End System Workflow & Data Flow Document

Document Version: 1.0
Firmware Version: FW-V1.0.0
Product Code: iotbowl
Model Code: ibw001
Platform: ESP32-S3 N16R8
Prepared For: Firmware, Backend, Frontend, Cloud

# 1. Summary

The PetWatch SmartBowl S3 is an IoT-enabled smart pet feeding system designed to:

- Measure bowl weight in real time

- Detect feeding behavior automatically

- Capture feeding images

- Upload telemetry and events to cloud infrastructure

- Store media in Azure Blob Storage

- Sync all operational data to the PetWatch backend ecosystem

The system uses:

- ESP32-S3 firmware

- MQTT for real-time communication

- HTTPS REST APIs for metadata sync

- Azure Blob Storage for image uploads

- MySQL backend aligned with PetWatch Master Schema

# 2. High-Level System Architecture

# 3. Device Hardware Overview

| Component | Description |
| --- | --- |
| MCU | ESP32-S3 N16R8 |
| Weight Sensor | HX711 + Load Cell |
| Camera | OV2640 |
| Display | SSD1306 OLED |
| Storage | SD_MMC |
| Connectivity | WiFi STA/AP + BLE |
| Cloud Protocols | MQTT + HTTPS |

# 4. Device Boot Workflow

## 4.1 Boot Sequence

# 5. MQTT Architecture

## 5.1 MQTT Base Topic

All topics follow PetWatch schema convention.

petwatch/{product_code}/{model_code}/{serial_number}

Example:

petwatch/iotbowl/ibw001/pw-ibw001-2602-00001

All topic names must remain lowercase.

## 5.2 Published Topics

| Topic | Purpose |
| --- | --- |
| /status | Device connection status |
| /telemetry | Live bowl telemetry |
| /event | Feeding events |
| /blob | Image upload events |
| /calibration | Calibration updates |
| /ota/status | OTA progress |

## 5.3 Subscribed Topics

| Topic | Purpose |
| --- | --- |
| /ack | Device UUID assignment |
| /cmd/tare | Remote tare command |
| /cmd/schedule | Feeding schedule push |
| /cmd/ota | OTA firmware trigger |

# 6. MQTT Connection Flow

# 7. Telemetry Workflow

Telemetry is published periodically.

Default interval:

5 seconds

## 7.1 Telemetry Data Flow

## 7.2 Telemetry Payload Example

{
  "msg_type": "telemetry",
  "product_code": "iotbowl",
  "model_code": "ibw001",
  "serial_number": "pw-ibw001-2602-00001",
  "fw_version": "FW-V1.0.0",
  "battery_level": 100,
  "battery_status": "full",
  "rssi": -42,
  "actual_weight": 152.3,
  "tracked_weight": 149.8,
  "planned_portion_size": 400,
  "portion_unit": "grams",
  "stability_status": "stable",
  "bowl_status": "consuming",
  "connection_status": "online",
  "sleep_status": "awake",
  "ota_status": "idle",
  "calibration_status": "completed",
  "seq": 120,
  "uptime_ms": 325001
}

# 8. Bowl State Machine

## 8.1 Bowl States

| Bowl State | Meaning |
| --- | --- |
| empty | Bowl empty |
| food_present | Food added |
| consuming | Pet actively eating |
| leftover | Food remains after feeding |

## 8.2 Detection Logic

Weight <= 5g
    → empty

Weight jump > threshold
    → food_present

Negative slope detected
    → consuming

Stable leftover weight
    → leftover

# 9. Feeding Session Workflow

Idle
  ↓
Food Selection
  ↓
Empty Bowl Confirmation
  ↓
Add Food
  ↓
Feeding Started Event
  ↓
Dispense Completed Event
  ↓
Dog Eating Detection
  ↓
Camera Burst Trigger
  ↓
Leftover Measurement
  ↓
Feeding Completed

## 9.1 Feeding Events

| Event Type | Meaning |
| --- | --- |
| feeding_started | Feeding session began |
| dispense_started | Food dispensing started |
| dispense_completed | Dispensing finished |
| eating_started | Pet started eating |
| leftover_measured | Leftover captured |
| feeding_completed | Session completed |
| feeding_failed | Session failed |

## 9.2 Feeding Event Payload

{
  "msg_type": "feeding_event",
  "event_type": "feeding_completed",
  "event_status": "success",
  "event_value": 346.2,
  "event_unit": "grams",
  "actual_served_value": 398.5,
  "leftover_value": 52.3,
  "consumed_value": 346.2,
  "portion_unit": "grams",
  "food_name": "Dry Kibble",
  "serial_number": "pw-ibw001-2602-00001"
}

# 10. Camera Workflow

## 10.1 Camera Burst Trigger Conditions

| Trigger | Description |
| --- | --- |
| eating_started | Automatic trigger |
| feed_confirm | Manual confirmation |
| feeding_completed | Final upload flush |

## 10.2 Camera Upload Flow

Capture JPEG
   ↓
Save to SD Card
   ↓
Build Azure Blob URL
   ↓
HTTPS PUT Upload
   ↓
Metadata POST to Backend
   ↓
Publish MQTT Blob Event

## 10.3 Blob Event Payload

{
  "msg_type": "blob_upload_event",
  "file_name": "feed_1234.jpg",
  "file_type": "image",
  "upload_status": "success",
  "blob_url": "https://azure.blob.core.windows.net/...",
  "db_status": "success",
  "file_size": 182340
}

# 11. Azure Blob Upload Flow

ESP32
  ↓
Read SD File
  ↓
HTTPS PUT
  ↓
Azure Blob Storage
  ↓
Return HTTP 201
  ↓
Metadata POST
  ↓
Backend DB Insert

# 12. Backend Ingestion Workflow

MQTT Message Received
   ↓
JSON Validation
   ↓
Device Lookup
   ↓
Map to Schema Tables
   ↓
Insert / Update Database
   ↓
Push Dashboard Updates

# 13. Database Mapping

| Payload Field | Database Table |
| --- | --- |
| actual_weight | bowl_current_status |
| tracked_weight | bowl_current_status |
| bowl_status | bowl_current_status |
| stability_status | bowl_current_status |
| event_type | feeding_event_log |
| feed_status | feeding_log |
| consumed_value | feeding_log |
| calibration_factor | bowl_calibration_history |
| fw_version | device_master |
| mqtt_base_topic | device_connectivity |

# 14. REST API Endpoints

| Method | Endpoint | Purpose |
| --- | --- | --- |
| GET | /status.json | Current device status |
| GET | /stream | SSE live stream |
| POST | /tare | Remote tare |
| POST | /wifi/save | Save WiFi credentials |
| POST | /wifi/clear | Clear WiFi |
| POST | /cal/reset | Reset calibration |

# 15. Frontend Dashboard Workflow

## Dashboard Features

- Live bowl weight

- Feeding history

- Camera snapshots

- Device online/offline status

- RSSI monitoring

- OTA status

- Calibration status

- Consumption analytics

# 16. Firmware Module Architecture

| File | Responsibility |
| --- | --- |
| Config.h | Configuration & constants |
| Pins.h | GPIO mapping |
| LoadSensor.h | HX711 processing |
| CameraSD.h | Camera & SD logic |
| MqttClient.h | MQTT communication |
| BowlLogic.h | Bowl state machine |
| Buttons.h | UI navigation |
| RestAPI.h | Local REST APIs |
| BleProvision.h | BLE provisioning |
| SmartBowl_S3.ino | Main firmware |

# 17. Final Workflow

ESP32 Device
    ↓
Read Sensors
    ↓
Process Weight
    ↓
Detect Bowl State
    ↓
Generate Events
    ↓
Capture Images
    ↓
Upload Blob
    ↓
Publish MQTT
    ↓
Backend Ingestion
    ↓
Database Storage
    ↓
Dashboard Visualization

# 20. Conclusion

The system provides:

- Real-time telemetry

- Feeding automation

- Intelligent bowl state tracking

- Media capture workflows

- Cloud-native architecture

- Backend-ready structured payloads

- Frontend-ready live data streams